|  | User Guide for IoT Codec Test Tool |  |  |  |
| --- | --- | --- | --- | --- |
|  | User Guide for IoT Codec Test Tool |  |  |  |
|  | Issue | 01 |  |  |
|  | Date | 2020-11-16 |  |  |
|  | HUAWEI TECHNOLOGIES CO., LTD. |  |  |  |

Copyright © Huawei Technologies Co., Ltd. 2020. All rights reserved. No part of this document may be reproduced or transmitted in any form or by any means without prior written consent of Huawei Technologies Co., Ltd. Trademarks and Permissions and other Huawei trademarks are trademarks of Huawei Technologies Co., Ltd. All other trademarks and trade names mentioned in this document are the property of their respective holders. Notice The purchased products, services and features are stipulated by the contract made between Huawei and the customer. All or part of the products, services and features described in this document may not be within the purchase scope or the usage scope. Unless otherwise specified in the contract, all statements, information, and recommendations in this document are provided "AS IS" without warranties, guarantees or representations of any kind, either express or implied. The information in this document is subject to change without notice. Every effort has been made in the preparation of this document to ensure accuracy of the contents, but all statements, information, and recommendations in this document do not constitute a warranty of any kind, express or implied.

| Huawei Technologies Co., Ltd. |  |
| --- | --- |
| Address: | Huawei Industrial Base Bantian, Longgang Shenzhen 518129 People's Republic of China |
| Website: | https://www.huawei.com |
| Email: | support@huawei.com |

Contents

1 Background	1

2 Prerequisites	3

3 Procedure	4

4 Fault Locating and Error Codes	9

# Background

This document describes the usage of the IoT codec test tool by using the demo included in the tool package as an example.

The following figure shows the process of connecting devices and applications to the IoT platform.

- Solution development: planning the project and deciding on devices to connect and how applications will be connected.

- Preparations: preparing necessary materials or resources, such as devices (modules) and the IoT platform environment.

- Development: developing product models based on device data, and developing a codec and applications.

- End to end (E2E) commissioning: performing E2E (from devices to applications) commissioning, including testing the basic functions on upstream and downstream data, and services and functions in different scenarios.

During the process, if the codec is not tested thoroughly in the development step, issues such as prolonged E2E commissioning and frequent reworks will occur. To prevent these issues, this document describes how to quickly locate faults, if any, in the codec to accelerate the E2E commissioning. PluginDetector is a tool used for local codec testing. It simulates the environment where the IoT platform invokes the codec for encoding and decoding, and tests items such as codec data format and the encoding and decoding function.

With PluginDetector, a codec testing step is added between the development and E2E commissioning.

# Prerequisites

- You have developed a product model.

- You have developed a codec by following the instructions in Huawei IoT Platform Encoding and Decoding Library Development Guide.

- JRE 1.8 has been installed.

- The data reported or command delivered is correct.

- The testing should be performed in an environment running a Windows operating system.

# Procedure

          - Preparing Files

- Obtain the IoT Codec Test Tool package.

- Decompress the package. Ensure that the files pluginDetector.jar, devicetype-capability.json, package.zip, and folder tool are in a same directory.

- tool is used to scan invalid methods.

- devicetype-capability.json is obtained from the profile directory of a product model.

- package.zip is an unsigned codec package.

- pluginDetector.jar is the test tool. Right-click it, choose Open with from the context menu, and select Java(TM) Platform SE binary to run this tool.

Note: Do not rename the files devicetype-capability.json, package.zip, and folder tool.

          - Testing

When the test tool is started, it will automatically check whether the data in the codec package and the product model is consistent, and whether there are any invalid packages, classes, and methods. You can also use the tool to check codec usage in the following three scenarios: data reporting, command delivery, and command execution result reporting.

Do not delete the two cache folders generated in the same directory as the tool when the tool is running. Each time the tool runs, the two cache folders will be automatically overwritten.

- Data reporting

  - Obtain the code stream reported by a device.

  - Choose the data report tab in the test tool and enter the code stream in hexadecimal format, for example, AA72000032088D0320623399.

  - Click start detect to view the decoded JSON data. If the decoded data and the message report data is success are displayed, the decoding is successful.

If ERROR messages are displayed, a decoding error occurs. Rectify the error by referring to 4 Fault Locating and Error Codes.

  - After the data reporting, the test tool invokes the encoding interface of the codec package to encode the response to the device. If the message encode ack result success is displayed, the encoding is successful.

- Command delivery

  - An application calls an API to deliver an asynchronous command to a device.

  - Choose the send command tab in the test tool, enter the command in JSON format, and click start detect. The test tool invokes the encoding interface to encode the command. If the message encode cmd result success is displayed, the encoding is successful.

{ 
"identifier": "123", 
"msgType": "cloudReq", 
"serviceId": "NBWaterMeterCommon", 
"cmd": "SET_DEVICE_LEVEL", 
"mid": 2016, 
"paras": { "value": "10" }, 
"hasMore": 0 
}

If ERROR messages are displayed, a decoding error occurs. Rectify the error by referring to 4 Fault Locating and Error Codes.

- Command execution result reporting

  - Obtain the code stream of the reported command execution result.

  - Choose the data report tab in the test tool and enter the code stream in the hexadecimal format, for example, AA7201000107E0.

  - Click start detect to view the decoded JSON data. If the decoded data and the message report command result success are displayed, the decoding is successful.

If ERROR messages are displayed, a decoding error occurs. Rectify the error by referring to 4 Fault Locating and Error Codes.

# Fault Locating and Error Codes

If an error occurs, for example, the configuration file is incorrectly configured, the input parameters are incorrect, or the decoding or encoding fails, rectify the error based on the error codes.

                - Error codes

| Error Code | Description | Handling Suggestion |
| --- | --- | --- |
| 1001 | The devicetype-capability.json file does not exist. | Check whether the devicetype-capability.json file is in the same directory of the test tool. |
| 1002 | The content of the devicetype-capability.json file is in incorrect format. | Check whether the content of the devicetype-capability.json file is in JSON format. |
| 1003 | The properties in the devicetype-capability.json file are incomplete. | Check whether there are missing fields in the devicetype-capability.json file. |
| 1004 | Failed to read the package.zip file. | Check whether the package.zip file is in the same directory of the test tool and whether the package name has been changed. |
| 1005 | The package-info.json file does not exist. | Check whether the package-info.json file is in the root directory of package.zip. |
| 1006 | The content of package-info.json is in incorrect format. | Check whether the content of the package-info.json file is in JSON format. |
| 1007-1016 | Some properties cannot be found in the package-info.json file. | Check whether there are missing properties or incorrect property names in the package-info.json file. |
| 1017 | Some fields cannot be found in MANIFEST.MF. | Check whether MANIFEST.MF has been modified and whether the Bundle-Name, Bundle-Version and Bundle-SymbolicName fields exist. |
| 1018 | MANIFEST.MF does not exist. | Check whether MANIFEST.MF is in the correct directory. |
| 1019 | The content of MANIFEST.MF is in incorrect format. | Check whether the content format of MANIFEST.MF is the same as that of the template. |
| 1020 | No .xml file can be found in the OSGI-INF directory. | Ensure that there is only one .xm file in the OSGI-INF directory and the file name extension is in lowercase. |
| 1021 | The content of the .xml file in the OSGI-INF directory is incorrect. | Check whether the .xm file contains the class and interface fields. |
| 1022 | Properties do not match. | The values of model in package-info.json and devicetype-capability.json must be the same. |
| 1023 | Properties do not match. | The values of deviceType in package-info.json and devicetype-capability.json must be the same. |
| 1024 | Properties do not match. | The values of manufacturerName in package-info.json and devicetype-capability.json must be the same. |
| 1025 | Properties do not match. | The value of packageType in the package-info.json file must be CIGPlugin. |
| 1026 | Properties do not match. | The values of Bundle-Name in the package-info.json and MANEFEST.MF files must be the same. |
| 1027 | Properties do not match. | The values of Bundle-Version in the package-info.json and MANEFEST.MF files must be the same. |
| 1028 | Properties do not match. | The values of fileName in the package-info.json and MANEFEST.MF files must be the same. |
| 1029 | Properties do not match. | The values of bundleName and SymbolicName in the MANEFEST.MF file must be the same. |
| 1030 | MANIFEST.MF contains the field Export-Package and its value. | The Export-Package field and its value are not allowed in MANIFEST.MF. |
| 1031 | Properties do not match. | The values of class and srcName in the .xml file in the OSGI-INFO directory must be the same. |
| 1032 | Properties do not match. | The ProtocolAdapterImpl.java file must be located in the com/manufactureName/model/deviceType path. The values of manufactureName, model, and deviceType must be the same as those in the product model. |
| 1033 | Properties do not match. | The value of platform in the package-info file must be linux. |
| 1034 | Properties do not match. | The value of specVersion in the package-info file must be 1.0. |
| 1035 | Properties do not match. | The values of version and bundleVersion in the package-info file must be the same. |
| 2001 | Initialization failed. | Locate the error by referring to logs. |
| 2002 | Decoding failed because serviceId does not exist. | Ensure that the value of serviceId in the return value of the codec is the same as that in the product model. |
| 2003 | The input parameters are not in JSON format during command delivery. | Check the input parameters. |
| 2004 | The command execution result is null. | Ensure that the input parameters are correct and the encoding is done correctly. |
| 3000 | The return value of the getManufacturerId() interface is incorrect. | Check whether the return value is the same as that in the devicetype-capability.json file. |
| 3001 | The return value of the getModel () interface is incorrect. | Check whether the return value is the same as that in the devicetype-capability.json file. |
| 3002 | Decoding failed. The return value is null. | Check the return value of the codec. |
| 3003 | Decoding failed. The value of msgType is null. | Check whether the return value of the codec contains the msgType field. |
| 3004 | Decoding failed. The value format of hasMore is incorrect. | The value of hasMore is not of the int type. |
| 3005 | Decoding failed. The input parameters are incorrect. | Ensure that data is in the input parameters. |
| 3006 | Decoding failed. The input parameters are incorrect. | The value of data must be arraylist. |
| 3007 | Decoding failed. The body format is incorrect. | The body must be an object. |
| 3008 | Decoding failed. The value of msgType is not deviceReq or deviceRsp. | The value of msgType must be deviceReq or deviceRsp. |
| 4001 | Failed to invoke the codec to encode the command response. | Check whether the class field in the .xml files in the jar and OSGI-INF directories of the codec is correctly set and whether the decoding method is correct. |
| 4002 | Failed to invoke the codec to encode the command. | Check whether the class field in the .xml files in the jar and OSGI-INF directories of the codec is correctly set and whether the encoding method is correct. |
| 4003 | Failed to invoke the codec to encode the ACK message. | Check whether the class field in the .xml files in the jar and OSGI-INF directories of the codec is correctly set and whether the encoding method is correct. |
| 4004 | Encoding failed. The value of the msgType field is not cloudReq. | Ensure that the msgType field is included in the input parameters and its value is cloudReq. |
| 4005 | Encoding failed. The serviceId field cannot be found. | Ensure that the serviceId field is included in the input parameters. |
| 4006 | Encoding failed. The cmd field cannot be found. | Ensure that the cmd field is included in the input parameters. |
| 4007 | Encoding failed. The hasMore field cannot be found. | Ensure that the hasMore field is included in the input parameters. |
| 4008 | Encoding failed. The mid field cannot be found. | Ensure that the mid field is included in the input parameters. |
| 4009 | Encoding failed. The return value is incorrect. | No serviceId or serviceData exists in the JSON node. |
| 4010 | Encoding failed. The date format is incorrect. | Ensure that the date format is correct. |
| 6001 | Invalid classes in the code. | Use secure classes. |
| 6002 | Invalid packages in the code. | Use secure .jar packages. |
| 6003 | Invalid functions in the code. | Use secure functions. |
| 100 | JAVA_HOME is not configured. | Configure JAVA_HOME in environment variables. |
| 101 | No jdeps.exe file can be found in the bin directory of the JDK. | This file is required by the test tool. Use JDK 8 or later versions. |